CADERNO UNIFICADO DE CHECKLISTS NORMAIS

Padronização Operacional da Frota (7 Frotas) — OMNI Táxi Aéreo

MÉTODOS DE EXECUÇÃO:  C/R = Challenge and Response (Desafio e Resposta)  |  R/D = Read and Do (Ler e Fazer - exclusivo para Before Start)
GATILHOS PADRONIZADOS: Altitude de tráfego (500 ft / 700 ft noturno) & Aproximações/Descidas a 5 NM

FROTA H-160

| PREFLIGHT (C/R) |  |
| --- | --- |
| Gatilho: Entrada do PM após briefing dos PAX. |  |
| Dtd | Card Inserted / Flap Closed |
| Collective Pitch | Locked |
| Static / Shed / Rlg | Condition Checked |
| Bat 1 + Bat 2 | On |
| Gen 1 + Gen 2 + Emerg. Gen | On |
| Lamp / Audio Test | Audible / All Illuminated |
| Eng. Oil Level and Temp | Checked |
| Dispatch | Checked |
| Fuel Quantity | Confirm ____ kg |
| Weight Data | Enter / Validated |
| Hdg | Compare with Stby Compass |
| Dh | 1000 ft |
| Altimeters (+Iesi) | Set / Xchecked |
| Htaws | On |
| Wx Radar | Stby |
| Oei Rating | Hi / Lo / Ct / Hi |
| Flight Controls | Checked / Then Coll. Locked |
| Wx Radar (If GPU Connected – Test) | Stby |
| Acas / Radios | Test All |
| Emrg. Exit | Armed |
| ► Checklist complete. Standby Before Start |  |

| BEFORE START (R/D) |  |
| --- | --- |
| Gatilho: Preflight completo, cabine/cockpit prontos e autorização obtida. |  |
| Anti Coll. / Lights | Red / On |
| Batt Voltage (> 23) | Check Voltage ____ |
| ► Checklist complete. Standby After Start |  |

| AFTER START (C/R) |  |
| --- | --- |
| Gatilho: Partida completa dos motores, parâmetros normais/estabilizados e GPU desconectada. |  |
| Gpu (If Connected) | Off / Disconnected |
| Rlg | Off / Guarded |
| Floats | Area Clear / Test / Off |
| Collective Pitch | Unlock / Reduce Min |
| Pre-Flight / RA Test | Perform |
| Collective Pitch | Reduce Min / Locked |
| ► Checklist complete. Standby Before Taxi |  |

| BEFORE TAXI (C/R) |  |
| --- | --- |
| Gatilho: After Start completo, portas/passageiros prontos e intenção de táxi. |  |
| Eng 1 + Eng 2 | Flight / Guarded |
| Master List / Dispatch | Checked |
| Atc | Cleared |
| Nose Wheel | Unlocked |
| Park Brake | Off |
| Brakes Test (Collective Full Down) | Perform |
| Hsi Hdg / Compass (After Turn) | Aligned |
| ► Checklist complete. Standby Before Takeoff |  |

| BEFORE TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Linha de parada/pista. Offshore: Helideck e passageiros prontos. |  |
| Eng 1 / Eng 2 | Flight / Guarded |
| Nr Hi | A/R |
| Dh | Adjust _____ ft |
| Fuel | Check Endurance |
| Float | A/R |
| Brake | A/R |
| Radar | A/R |
| Alt. A | Adjusted |
| Transponder | Activated |
| Ldg Anti Coll | White |
| Light | A/R |
| Alph | Thumbs Up |
| Takeoff Briefing | Perform |
| ► Checklist complete. Standby After Takeoff |  |

| AFTER TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Cruzando 500 ft AGL e livre de tráfego/obstáculos. |  |
| Radar | On |
| Nose Wheel | Unlocked |
| L/G | Up |
| Park Brake | Off |
| Compass | MG |
| Nr Hi | Off |
| ► Checklist complete. Standby Cruise |  |

| CRUISE (C/R) |  |
| --- | --- |
| Gatilho: Nivelado na altitude de cruzeiro e estabilizado em rota. |  |
| Afcs Upper Modes | Coupled |
| Htaws Mode | Set |
| Ldg Light | Off |
| Fuel Quantity | Monitor |
| ► Checklist complete. Standby Descent |  |

| DESCENT (C/R) |  |
| --- | --- |
| Gatilho: A 5 NM do TOD (FMS) ou autorização de descida ATC. |  |
| Altimeters (+Iesi) | Set |
| Da / Dh | Adjust _____ ft |
| Htaws Mode | Set A/R |
| Cr. Ht | Set A/R |
| Ldg Light | On |
| ► Checklist complete. Standby Landing |  |

| LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore/Offshore: A 5 NM do destino, na altitude de tráfego. |  |
| Nr Hi | A/R |
| Dh | A/R |
| Dg | Set Only Offshore |
| Park Brake | Set A/R |
| L/G | Down (3 Greens) |
| Nose Wheel | A/R |
| Float | A/R |
| Radar | A/R |
| Alt. A | Adjust _____ ft |
| Landing Briefing | Perform |
| Landing Area | Green Deck |
| ► Checklist complete. Standby After Landing |  |

| AFTER LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Pista livre. Offshore: Pousado, coletivo embaixo e equalizado. |  |
| Afcs Upper Mode | Disengage |
| Flight Controls | Center / Full Down |
| Collective Pitch | A/R |
| Floats | Off |
| Radar / Transponder | Stnd-By |
| Anti Coll | A/R |
| ► Checklist complete. Standby Shut Down |  |

| SHUT DOWN (C/R) |  |
| --- | --- |
| Gatilho: Aeronave estacionada, freio aplicado e calços colocados. |  |
| Collective Pitch | Locked |
| Park Brake | On |
| Cyclic / Pedals | Centered |
| Eng 1 / Eng 2 | (Idle 2 min) |
| Ecs | Off |
| Eng 1 / Eng 2 | Off |
| ► Checklist complete. |  |

FROTA S-92

| PREFLIGHT (C/R) |  |
| --- | --- |
| Gatilho: Tripulação técnica sentada e ajustada no cockpit (pós-scanflow). |  |
| Cockpit Preparation & Scan Flow | Completed |
| Parking Brake | Reset / On |
| EFBs / Documents | Onboard |
| ► Checklist complete. Standby Before Start |  |

| BEFORE START (R/D) |  |
| --- | --- |
| Gatilho: Preflight completo, preparação de cabine/cockpit finalizada e autorização obtida. |  |
| Battery Switch | On |
| Fuel Qty | Check |
| HUMS Data Card | Installed |
| Ext Lights | A/R |
| Fire Det | Test |
| APU Control & APU Gen | On |
| ECS | Vent |
| MFDs Overlays | Adjusted |
| RTUs | Tuned |
| FMS, FM, WX Radar | On / Stby |
| Backup Att Ind | Test Then Arm |
| Fire Det, PB Lamp, Flt Ctrl, AFCS, LDI, CVR/FDR, EGPWS, TCAS | Test |
| Flotation, Life Raft (DSC) | Test |
| MGB Pressure System (OBE) | Check |
| Aux Fuel Panel | All Open and Off |
| ► Checklist complete. Standby After Start |  |

| AFTER START (C/R) |  |
| --- | --- |
| Gatilho: Partida completa dos motores, parâmetros normais/estabilizados e GPU desconectada. |  |
| Throttles and Fuel | Fly / Direct |
| AC GEN #2 (2 Sec) Then #1 | On |
| VIB CTRL | On |
| APU Ctrl – (If SID IMC) | Off / On |
| Air Source Heat | Eng |
| ► Checklist complete. Standby Before Taxi |  |

| BEFORE TAXI (C/R) |  |
| --- | --- |
| Gatilho: After Start completo, portas/passageiros prontos e intenção de táxi. |  |
| Cockpit Setup (MFD/DCP/TCAS/NAV-AIDS) | Set / Xcheck |
| Emerg / Heels Lights | Arm |
| Backup Att Ind | Uncage |
| EICAS Parameters | Green and Fuel Qty |
| Altimeters, FMS Flight Plan | Set / Xcheck |
| Door / Ramp | Secured |
| Compass / Heading | Slaved / Xcheck |
| Area / Chocks | Clear / Removed / Thumbs Up |
| Takeoff Briefing – (VFR / IFR) | Completed |
| Taxi Clearance and Transponder | Request |
| Cabin Lights | A/R |
| Landing Lights | On |
| HISL Strobe Lts | A/R |
| Parking Brake | Released, No Advisory |
| ► Checklist complete. Standby Before Takeoff |  |

| BEFORE TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Linha de parada/pista. Offshore: Helideck e passageiros prontos. |  |
| Throttles and Fuel | Fly & Direct |
| Heading Bugs / Course Selectors | Set |
| Cautions / Warnings / Advisories | Checked |
| AFCS | All Green / Att / FD #1 or #2 |
| Weather Radar | On / Test (A/R) |
| Flight Attendant Briefing | A/R |
| Door / Ramp (Offshore) | Secured |
| Area / Chocks (Offshore) | Clear / Thumbs Up |
| AC GEN #2 Then #1 / VIB CTRL | On |
| APU Ctrl | A/R |
| NAV AIDS / DCP (All References) | Set |
| All EICAS Parameters | Green and Fuel Qty |
| Compass / Heading | Xcheck |
| Takeoff Briefing | Completed |
| Floats | Arm |
| Anti-Coll, Pos. and Landing Lights | On |
| ► Checklist complete. Standby After Takeoff |  |

| AFTER TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Cruzando 500 ft AGL e livre de tráfego/obstáculos. |  |
| Landing Gear | Up |
| FD | A/R |
| Search Light | Off / Centered |
| Floats Safe | ≤ 80 KIAS |
| Parking Brake | A/R |
| Compass / Headings | A/R |
| HISL Strobe Lts | A/R |
| WX Radar | A/R |
| ► Checklist complete. Standby Cruise |  |

| CRUISE (C/R) |  |
| --- | --- |
| Gatilho: Nivelado na altitude de cruzeiro e estabilizado em rota. |  |
| Landing Lights / APU | A/R / Off |
| Altimeters | Set / Xcheck |
| Compass / Headings | Slaved / Xcheck |
| EPAC | Perform |
| Cruise Power | Set |
| VNAV | Set |
| Aux Fuel Mode Select | IAFS A/R |
| EICAS Parameters | Green and Fuel Qty |
| ► Checklist complete. Standby Descent |  |

| DESCENT (C/R) |  |
| --- | --- |
| Gatilho: A 5 NM do TOD (FMS) ou autorização de descida ATC. |  |
| ATIS and Clearance | Check |
| Descent / Approach Briefing (IMC - APU) | Completed |
| FMS / DCPs / Nav Aids | Set / Xcheck |
| EICAS Parameters | Green and Fuel Qty |
| Flight Attendant (ETA) | A/R |
| Landing Lights | On |
| Altimeters | QNH Set / Xcheck |
| ► Checklist complete. Standby Landing |  |

| LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore/Offshore: A 5 NM do destino, na altitude de tráfego. |  |
| Landing Gear | Down, 3 Green No Red |
| Parking Brake | Check Release, No Advisory |
| Flight Attendant Briefing | A/R |
| Landing Briefing | Completed |
| RWY / Landing Site | In Sight / Unit Confirmation |
| Green Deck (Offshore) | Confirmed |
| APU Ctrl | A/R |
| Floats | Arm |
| ► Checklist complete. Standby After Landing |  |

| AFTER LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Pista livre. Offshore: Pousado, coletivo embaixo e equalizado. |  |
| APU Ctrl & APU Gen / Int & Search Lts | On / A/R |
| WX Radar | Stby |
| DISC Horizontal | Set |
| Floats | Safe |
| APU On | Check |
| VIB CTRL | Off |
| Electric Conv | Done |
| Throttles | Idle |
| Anti-Coll, Pos. and Landing Lights | Off |
| Chocks | Apply |
| ► Checklist complete. Standby Shut Down |  |

| SHUT DOWN (C/R) |  |
| --- | --- |
| Gatilho: Aeronave estacionada, freio aplicado e calços colocados. |  |
| Parking Brake | Apply |
| LDG / Emerg / Heels / Interior Lights | Off |
| APU On | Check |
| VIB CTRL | Off |
| Electric Conv | Done |
| Throttles (2 Min) | Idle |
| Fire Det | Test |
| FMS, FM and WX Radar | Off |
| AUX FUEL Mode Select | Off |
| Backup Att Ind | Off Then Cage |
| Throttles | Stop |
| Fuel Control Switch #1 & #2 | Off |
| Rotor Brake | Apply |
| EICAS Parameters | Note Fuel Qty |
| ECS | Vent / Off |
| HUMS | Crew Change |
| APU Gen & APU Ctrl | Off |
| Interior Lights | Off |
| Battery | Off |
| Chocks | Apply |
| ► Checklist complete. |  |

FROTA AW189

| PREFLIGHT (C/R) |  |
| --- | --- |
| Gatilho: Tripulação técnica sentada e ajustada no cockpit (pós-scanflow). |  |
| Seat Belts / Life Jackets | Fasten |
| Circuit Breakers | All In |
| Rotor Brake | Off / Check CAS |
| Static Sources | Guarded |
| Fire Panel | Check |
| Center Console Switches | Off / Normal |
| Skytrac / CPI Controller | Norm / Guarded |
| Emerg. Float Panel | Off |
| Ldg Gear / Parking Brake | Down / On |
| Coll Switches | Guarded |
| ► Checklist complete. Standby Before Start |  |

| BEFORE START (R/D) |  |
| --- | --- |
| Gatilho: Preflight completo, preparação de cabine/cockpit finalizada e autorização obtida. |  |
| Bat Master | On |
| APU | Start |
| Main Batt | On |
| Emerg Light | Test / Arm |
| ECDU | Check / Stat |
| ECDU Lights | A/R |
| Pitot Heater / Camera / Wiper | Auto / On / A/R |
| Ldshare | A/R |
| EMS Power | As Req'd |
| Fire Panel | Check |
| AFCS Panel | APs Off |
| Ldg Gear / Nose Wheel / Prkg Brake | 3 Green / Locked / On |
| Nose Wheel Locked Pin | In Sight |
| ECDU Tests | Complete |
| ECDU Hyd | Flt Ctl Check |
| Floats | Test |
| Radar / TCAS / TAWS | Set |
| ATC (Clearance) | Obtained |
| MCDU / Perf Init / FPL | Perform |
| Fuel Qty | Check |
| MFD | Power Plant |
| ITT | < 150°C |
| ECDU Fuel | On / Open / Xfeed |
| ► Checklist complete. Standby After Start |  |

| AFTER START (C/R) |  |
| --- | --- |
| Gatilho: Partida completa dos motores, parâmetros normais/estabilizados e GPU desconectada. |  |
| External Power Source | Disconnect |
| APU | Off |
| AFCS | Test |
| Fuel Pump / Xfeed | Test |
| Altimeters | Set QNH |
| Rad Alt / DH | Test / Set 200 ft |
| Nav / Fuel Qty | Set / Check |
| HSI / Compass | Check |
| Powerplant | All Green / No CAS |
| ► Checklist complete. Standby Before Taxi |  |

| BEFORE TAXI (C/R) |  |
| --- | --- |
| Gatilho: After Start completo, portas/passageiros prontos e intenção de táxi. |  |
| Doors | Closed |
| Takeoff Briefing | Perform |
| Engine Mode Selectors | Flt / Nr 102% |
| T's & P's | Check |
| AP 1 & 2 | Engaged / FD Set |
| Alt Selector | Set |
| APU | Ready |
| Air Cond / Heater | Set As Req'd / Check SOV |
| ATC Clearance | Obtain |
| Nose Wheel | Unlock |
| Parking Brake | Off |
| Wheel Brakes | Check |
| ► Checklist complete. Standby Before Takeoff |  |

| BEFORE TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Linha de parada/pista. Offshore: Helideck e passageiros prontos. |  |
| Eng Mode Selectors | Flt / Nr 102% |
| MFD | PF FPL / PM Powerplant |
| AP | ATT / FD Set |
| Fuel Qty | Check |
| AEO Limiter | A/R |
| XPDR / TCAS | TA / RA |
| Lights | Set As Req'd |
| Nose Wheel Lock | Locked |
| Floats (Over Water) | Armed |
| HLO / ATC | Cleared |
| Takeoff Briefing | Perform |
| ► Checklist complete. Standby After Takeoff |  |

| AFTER TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Cruzando 500 ft AGL e livre de tráfego/obstáculos. |  |
| Ldg Gear | Up |
| Parking Brake | Off |
| AEO Limiter | Off |
| ► Checklist complete. Standby Cruise |  |

| CRUISE (C/R) |  |
| --- | --- |
| Gatilho: Nivelado na altitude de cruzeiro e estabilizado em rota. |  |
| Ldg Light | Off |
| Power Plant | All Green / No CAS |
| Pitot Htr / Anti-Ice | Check / Set |
| Altimeter / DH | Set / Adjust |
| Radios / Xponder | Crosscheck |
| Nav Aids / FD Modes | Set / Check |
| Fuel Qty | Check Every 30 min |
| ► Checklist complete. Standby Descent |  |

| DESCENT (C/R) |  |
| --- | --- |
| Gatilho: A 5 NM do TOD (FMS) ou autorização de descida ATC. |  |
| Briefing | Perform |
| Nav Aids | Set |
| Altimeter / DH | Set / Adjust |
| Ldg Light | As Req'd |
| TAWS / Radar | As Req'd |
| Alt. A | Set / Engage |
| ► Checklist complete. Standby Landing |  |

| LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore/Offshore: A 5 NM do destino, na altitude de tráfego. |  |
| Ldg Gear / Nose Wheel | Down / 3 Green / Locked |
| Parking Brake | A/R |
| Powerplant | Check / CAS |
| Ldg Light / AEO Lim | On / As Req'd |
| DH | Set LDP |
| Landing Briefing | Perform |
| ► Checklist complete. Standby After Landing |  |

| AFTER LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Pista livre. Offshore: Pousado, coletivo embaixo e equalizado. |  |
| Nose Wheel / Park Brake | As Req'd |
| XPDR / WX Radar | As Req / Stby |
| Floats | As Req / Stby |
| Lights | As Req'd |
| APU | As Req |
| ► Checklist complete. Standby Shut Down |  |

| SHUT DOWN (C/R) |  |
| --- | --- |
| Gatilho: Aeronave estacionada, freio aplicado e calços colocados. |  |
| Flight Controls | Centered / MPOG |
| Nose Wheel / Parking Brake | Locked / On |
| AFCS | Off |
| MFD | Power Plant |
| Eng Mode Switches | Idle / Time (2 min) |
| WX Radar / VHF FM / Emerg Lights | Off / Off / Off |
| APU | Confirm On |
| Eng Mode Selector (After 2 min) | Off |
| Fuel Xfeed / Pumps 2 | Closed / Off |
| Rotor Brake (≤ 40% Nr) | Apply |
| Air Cond / Fan | Off |
| Internal Lights / Anti-Coll | Off |
| Fuel Qty | Record |
| APU | Off & Wait Cldw (1 min) |
| Main Batt | Off |
| Batt Master | Off |
| ► Checklist complete. |  |

FROTA H145

| PREFLIGHT (C/R) |  |
| --- | --- |
| Gatilho: Tripulação técnica sentada e ajustada no cockpit (pós-scanflow). |  |
| Exterior Check | Completed |
| Cards / Life Vests / Headsets | Checked |
| Seat and Pedals / Belt | Adjust / Fastened |
| First Aid Kit | Stowed |
| Fire Extinguishers | Secured |
| Loose Items | Stowed |
| Pins (3) and Covers | Stowed |
| Overhead Panel | Completed |
| Instrument Panel | Completed |
| Center Console | Completed |
| Coll. Pitch Levers | Latched |
| Rotor Brake | Released |
| ► Checklist complete. Standby Before Start |  |

| BEFORE START (R/D) |  |
| --- | --- |
| Gatilho: Preflight completo, preparação de cabine/cockpit finalizada e autorização obtida. |  |
| BAT MST (20sec system boot) | On |
| VMS | Press NUM |
| Weight | Set / Validated |
| Master List | Power-Up Test OK |
| Iso Call | A/R |
| Passenger Advisory Lights | A/R |
| Fire SYS 1 + 2 | Tested |
| LAMP | Tested |
| IBF Open (10 sec) | Tested & Normal |
| ADELT TST (On 24h duty) | Completed |
| Night Flight (EMS) | Prepared |
| Master List | Checked |
| Flight Controls | Checked |
| DC Volts | ≥ 23.5V |
| Fuel Quantity | Sufficient |
| ATIS + BARO (3) | Note and Set |
| Start Clearance | Received |
| ► Checklist complete. Standby After Start |  |

| AFTER START (C/R) |  |
| --- | --- |
| Gatilho: Partida completa dos motores, parâmetros normais/estabilizados e GPU desconectada. |  |
| OVHD SW's | Set A/R |
| Air Conditioning (ECS) | On - Ckpt / Cab |
| Avionics | GTN Check and Set |
| Coll. / MFD 4 | Unlatch / Syst |
| LAMP | Check: MKR, DME, NMS |
| P-FLT TST (IFR flight) | P-FLT Test OK |
| Floats ARMED | Tested |
| WXR (IFR flight) | Tested, then STBY |
| ► Checklist complete. Standby Before Taxi |  |

| BEFORE TAXI (C/R) |  |
| --- | --- |
| Gatilho: After Start completo, portas/passageiros prontos e intenção de táxi. |  |
| Doors | Closed |
| Fuel Quantity | Sufficient |
| Flight Plan | Set / Load |
| Collective | Unlatched |
| ENG 1/2 Main Sw's | Flight / Closed |
| WRN / Cautions | Clear |
| IESI | Aligned, IAS and ALT Valid |
| NR / N2 | VMS & FND Normal Range |
| MFD's | A/R |
| ALT.A / HDG / Course | Set |
| AP / BKUP SAS | On |
| Landing / Ext. Lights | A/R |
| Takeoff Clearance | Confirmed |
| Takeoff Briefing | Performed |
| ► Checklist complete. Standby Before Takeoff |  |

| BEFORE TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Linha de parada/pista. Offshore: Helideck e passageiros prontos. |  |
| Upper Modes | A/R |
| WXR | A/R |
| Landing / Ext. Lights | A/R |
| ► Checklist complete. Standby After Takeoff |  |

| AFTER TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Cruzando 500 ft AGL e livre de tráfego/obstáculos. |  |
| Upper Modes | A/R |
| WXR | A/R |
| Landing / Ext. Lights | A/R |
| ► Checklist complete. Standby Cruise |  |

| CRUISE (C/R) |  |
| --- | --- |
| Gatilho: Nivelado na altitude de cruzeiro e estabilizado em rota. |  |
| Altimeter | Check |
| FLI / IAS / VNE | Check |
| Flight Plan / V.CALC (GTN) | Set |
| Fuel Plan (GTN) | Check |
| Power Check (EPC) | A/R |
| ► Checklist complete. Standby Descent |  |

| DESCENT (C/R) |  |
| --- | --- |
| Gatilho: A 5 NM do TOD (FMS) ou autorização de descida ATC. |  |
| Flight Plan / NAVD / FND | Set / X-Check |
| Altimeter / ALT.A | Set |
| DA / DH | Set |
| Descent Briefing | Completed |
| IAS / ALT.A | Engaged |
| ► Checklist complete. Standby Landing |  |

| LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore/Offshore: A 5 NM do destino, na altitude de tráfego. |  |
| WXR | A/R |
| WRN / Cautions | Check |
| Landing / Search Light | On |
| Landing Briefing | Perform |
| ► Checklist complete. Standby After Landing |  |

| AFTER LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Pista livre. Offshore: Pousado, coletivo embaixo e equalizado. |  |
| Cyclic and Pedals | Neutral |
| Collective | Latched |
| Landing / Search Light | Off / Stowed |
| ► Checklist complete. Standby Shut Down |  |

| SHUT DOWN (C/R) |  |
| --- | --- |
| Gatilho: Aeronave estacionada, freio aplicado e calços colocados. |  |
| ENG 1 / 2 | Idle (Start Clock) |
| MFD 4 | VMS |
| All Consumers (Expt A-Coll & BLT/SMK) | Off |
| ENG 1 / 2 (After 30 sec) | Off / TOT Monitor |
| Rotor Brake (Nr < 50%) | Apply |
| Rotor Brake (After rotor stop) | Release |
| A-Coll / BLT / SMK | Off |
| Flight Report | Validated |
| FND ML | Download Complete |
| BAT MSTR | Off |
| ► Checklist complete. |  |

FROTA H175

| PREFLIGHT (C/R) |  |
| --- | --- |
| Gatilho: Tripulação técnica sentada e ajustada no cockpit (pós-scanflow). |  |
| Pedals and Seats | Adjust |
| Access Doors | Check |
| Jettison Handles | Check |
| Fire Extinguisher | In Place / Check |
| Collective Pitch | Latched |
| Shed Switch | Off |
| Circuit Breakers | All Set |
| Static Port Valves | Normal |
| Rotor Brake | Released |
| Standby Compass | Check |
| FMS Switches | Off |
| Ice Sensor / Windshield De-Ice | Off |
| Floats | Off |
| Radio Altimeters 1&2 | Off |
| ACAS | Armed |
| HTAWS | Standby |
| Hydraulic Auxiliary Pump | Auto |
| Wipers Control Box | Off |
| ECS | Off |
| Skytrac | Normal |
| HUMS Card | Check |
| Fuel Pumps | Off |
| Engine Control Panel | Off / SOV Normal |
| Fire Exting | Guarded Engine |
| Training | Flight |
| Weather Radar | Off |
| Marine VHF | Off |
| CPI | Guarded / Lights Off |
| Landing Gear Panel | Down / Emerg Off |
| Parking Brake | Applied |
| Hydraulic Bypass 1&2 | Normal |
| Emergency Cut Off 1&2 | On |
| Generator 1&2 / Battery 1&2 | Off |
| Emergency Generator / Alternator | Off |
| Lights Control Panel & Cabin Signs | All Off |
| Mission Selector | Off |
| ELT | Armed |
| ► Checklist complete. Standby Before Start |  |

| BEFORE START (R/D) |  |
| --- | --- |
| Gatilho: Preflight completo, preparação de cabine/cockpit finalizada e autorização obtida. |  |
| Battery 1&2 / Generator 1&2 | On |
| EPU | As Required |
| Emergency Exit Switch | On, Then Arm |
| VMS Subpages | Check |
| Fuel Quantity | Check |
| Weight Data | Enter |
| Power-Up Test | OK |
| Hydraulic Auxiliary Pump | Test, Then Auto |
| Pre Start Test OK | Check |
| Collective Pitch | Latched |
| Cyclic Stick | Centered |
| FMS 1&2 | On |
| Radio Altimeters 1&2 | On |
| ATC Clearance | Obtain |
| Altimeter | Set |
| ► Checklist complete. Standby After Start |  |

| AFTER START (C/R) |  |
| --- | --- |
| Gatilho: Partida completa dos motores, parâmetros normais/estabilizados e GPU desconectada. |  |
| EPU Switch | Off / Disconnected |
| Generator 1 & 2 Parameters | Normal Range |
| Emergency Generator | On |
| Alternator (If Installed) | On |
| Ice Sensor / Windshield (If Installed) | As Required |
| HTAWS | On |
| Lamp Switch | Test |
| Fuel Pumps | Test |
| ECS | As Required |
| Backup Radio | Test |
| Weather Radar | Standby |
| Marine VHF | On |
| CPI | Guarded / Lights Off |
| Collective | Unlatched |
| P. FLT Switch | Test |
| P. FLT Test OK | Check |
| AP 1/2 | On |
| Position Lights | On |
| ► Checklist complete. Standby Before Taxi |  |

| BEFORE TAXI (C/R) |  |
| --- | --- |
| Gatilho: After Start completo, portas/passageiros prontos e intenção de táxi. |  |
| Takeoff Briefing | Perform |
| Landing Light | On |
| Parking Brake | Released |
| Nose Wheel | Unlocked |
| Wheel Brakes (During Taxi) | Check |
| HSI | Marks Increasing / Decreasing |
| Alt. A | Set |
| ► Checklist complete. Standby Before Takeoff |  |

| BEFORE TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Linha de parada/pista. Offshore: Helideck e passageiros prontos. |  |
| Anti Collision | White |
| Transponder | On |
| Floats | Test / Auto |
| CWP & Master List | Check |
| Nose Wheel | Locked |
| Engines 1&2 (Offshore) | Flight & Guarded |
| Weight Data | Entered |
| Alt. A | Set |
| External Lights | On |
| Cabin Sign | On |
| Landing Lights | On |
| Helideck Area | Clear |
| Collective | Unlatched |
| Takeoff Briefing | Perform |
| ► Checklist complete. Standby After Takeoff |  |

| AFTER TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Cruzando 500 ft AGL e livre de tráfego/obstáculos. |  |
| Landing Gear | Up / Lights Off |
| Weather Radar | On |
| Parking Brake | Released |
| Compass | MG |
| HTAWS | Normal |
| ► Checklist complete. Standby Cruise |  |

| CRUISE (C/R) |  |
| --- | --- |
| Gatilho: Nivelado na altitude de cruzeiro e estabilizado em rota. |  |
| Fuel Quantity | Check (Every 30 min) |
| Landing Light | Off |
| Alt. A / Cr. Ht | Set |
| ► Checklist complete. Standby Descent |  |

| DESCENT (C/R) |  |
| --- | --- |
| Gatilho: A 5 NM do TOD (FMS) ou autorização de descida ATC. |  |
| Landing Lights | On |
| Weather Radar | As Required |
| ► Checklist complete. Standby Landing |  |

| LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore/Offshore: A 5 NM do destino, na altitude de tráfego. |  |
| Landing Gear | Down / 3 Greens |
| Parking Brake | Released |
| Nose Wheel | Locked |
| DH / DA | Check |
| Landing Briefing | Perform |
| Compass (Offshore) | DG |
| Weather Radar (Offshore) | Standby |
| HTAWS (Offshore) | Low Alt |
| Landing Site (Offshore) | Green Deck |
| Traffic Pattern (Offshore) | Unit Confirmed |
| ► Checklist complete. Standby After Landing |  |

| AFTER LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Pista livre. Offshore: Pousado, coletivo embaixo e equalizado. |  |
| Nose Wheel | Unlocked |
| Transponder | Standby |
| Anticolision Light | Red |
| Floats | Off |
| Collective (Offshore) | Latched |
| Cyclic Stick & Pedals (Offshore) | Centered |
| Landing Lights (Offshore) | Off |
| Cabin Sign (Offshore) | Off |
| External Lights (Offshore) | Off |
| ► Checklist complete. Standby Shut Down |  |

| SHUT DOWN (C/R) |  |
| --- | --- |
| Gatilho: Aeronave estacionada, freio aplicado e calços colocados. |  |
| Collective Pitch | Latched |
| Parking Brake | Applied |
| Cyclic Stick & Pedals | Centered |
| Landing Lights | Off |
| Engine Control Switch | Idle (1 min) |
| AFCS | AP & SAS Off |
| Ice Sensor / Windshield | Off |
| HTAWS | Standby |
| Wiper Control Box | Off |
| Weather Radar | Off |
| Marine VHF | Off |
| ECS | Off |
| Fuel Pumps | Off |
| Engine Control Switches | Off |
| FMS 1 & 2 | Off |
| Radar Altimeters 1&2 | Off |
| Rotor Brake | Apply (< 50%) |
| Cabin Signs | Off |
| External Lights | Off |
| Cabin Emergency & Cockpit Lights | Off |
| Emergency Generator / Alternator | Off |
| Generator 1 & 2 | Off |
| Hydraulic Auxiliary Pump | Off |
| HUMS | Download Complete |
| Battery 1 & 2 | Off |
| ► Checklist complete. |  |

FROTA AW139

| PREFLIGHT (C/R) |  |
| --- | --- |
| Gatilho: Tripulação técnica sentada e ajustada no cockpit (pós-scanflow). |  |
| Circuit Breakers | Checked |
| ECL | Flight |
| Fire Extinguisher Panel | Btl Set Center |
| Emerg Floats Switches | Guarded |
| ► Checklist complete. Standby Before Start |  |

| BEFORE START (R/D) |  |
| --- | --- |
| Gatilho: Preflight completo, preparação de cabine/cockpit finalizada e autorização obtida. |  |
| Electrical Switches | A/R |
| MFD & CAS | Checked |
| Fuel Qty | Cross-Checked |
| Nose Wheel / Pedals | Locked / Centralized |
| Flight Controls | Checked |
| Test Control Panel | Checked |
| Emerg Floats | No Float on CAS / Checked |
| Eng Beep Trim | Checked |
| Eng Gov Switches | Auto |
| Main Bus Voltage | Above 23V |
| ATC Clearance | Obtain |
| Altimeters Setting | Set / Cross-Checked |
| Fuel Panel | Set |
| ► Checklist complete. Standby After Start |  |

| AFTER START (C/R) |  |
| --- | --- |
| Gatilho: Partida completa dos motores, parâmetros normais/estabilizados e GPU desconectada. |  |
| Electrical Switches | A/R |
| Center Console | A/R |
| MCDU Preflight | Completed |
| Engine Mode Switches | Flight |
| Fuel Xfeed / Fuel Pumps | Tested |
| AFCS | Tested / AP1 & AP2 On / ATT |
| Power Plant | All Green / Normal CAS |
| ► Checklist complete. Standby Before Taxi |  |

| BEFORE TAXI (C/R) |  |
| --- | --- |
| Gatilho: After Start completo, portas/passageiros prontos e intenção de táxi. |  |
| ATC Clearance | Obtained |
| Nose Wheel | Unlocked |
| Park Brk | Off |
| ► Checklist complete. Standby Before Takeoff |  |

| BEFORE TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Linha de parada/pista. Offshore: Helideck e passageiros prontos. |  |
| HLO | Thumbs Up |
| Power Plant | All Green / Normal CAS |
| Take Off Briefing | Performed |
| ATC Clearance | Obtained |
| Xponder / TCAS | TA-RA |
| External Lights | On |
| ► Checklist complete. Standby After Takeoff |  |

| AFTER TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Cruzando 500 ft AGL e livre de tráfego/obstáculos. |  |
| RPM Switch (≤ 90kt) | 100% Nr |
| Ldg Gear | A/R |
| Park Brk | A/R |
| Compass | MAG |
| DH | Set _____ ft |
| ► Checklist complete. Standby Cruise |  |

| CRUISE (C/R) |  |
| --- | --- |
| Gatilho: Nivelado na altitude de cruzeiro e estabilizado em rota. |  |
| Altimeter Setting | Set |
| External Lights | A/R |
| Power Plant | All Green / Normal CAS |
| ► Checklist complete. Standby Descent |  |

| DESCENT (C/R) |  |
| --- | --- |
| Gatilho: A 5 NM do TOD (FMS) ou autorização de descida ATC. |  |
| Helideck Contact | Done |
| Altimeter Setting | Set |
| Alt Sel | Set _____ ft |
| External Lights | A/R |
| EGPWS / Radar | A/R |
| Approach Briefing | Performed |
| ► Checklist complete. Standby Landing |  |

| LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore/Offshore: A 5 NM do destino, na altitude de tráfego. |  |
| Ldg Gear | Down / 3 Greens |
| Park Brk | A/R |
| Compass | DG |
| DH | Set LDP |
| Floats | A/R |
| Landing Briefing | Performed |
| Power Plant | All Green / No CAS |
| Landing Area | Green Deck |
| ► Checklist complete. Standby After Landing |  |

| AFTER LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Pista livre. Offshore: Pousado, coletivo embaixo e equalizado. |  |
| Nose Wheel / Park Brk | Unlocked / Off |
| RPM Switch | 100% Nr |
| Floats | Off |
| Radar / Xponder | Stby |
| External Lights | A/R |
| ► Checklist complete. Standby Shut Down |  |

| SHUT DOWN (C/R) |  |
| --- | --- |
| Gatilho: Aeronave estacionada, freio aplicado e calços colocados. |  |
| Nose Wheel / Park Brk | Locked / On |
| Flight Controls | Centered / MPOG |
| Bus Tie | On |
| Eng Mode Switches | Idle / Timing |
| Center Console | A/R |
| Fuel Panel | A/R |
| Eng Mode Switches (After 1 min) | Off |
| Eng Fuel (≤ 10% Ng) | Off |
| Rotor Brake (≤ 40% Nr) | Apply A/R |
| External Lights | Off |
| Fuel Qty | Noted |
| Electrical Switches | A/R |
| ► Checklist complete. |  |

FROTA H225

| PREFLIGHT (C/R) |  |
| --- | --- |
| Gatilho: Tripulação técnica sentada e ajustada no cockpit (pós-scanflow). |  |
| Overhead Panel | Checked |
| Engine Panel | Checked |
| Emerg Fuel Shutoff Levers | Fwd & Snap Wired |
| Arm Lever (Rotor Brake) | After |
| Rotor Brake (If Wind < 30kt) | Forward |
| Standby Compass | Checked |
| Clocks | Set |
| Sub Panel | Checked |
| Static Sources | Norm & Snap Wired |
| Landing Gear / Safety Pin | Down / Removed |
| Collective & AP Hyd Cut-Out | Locked / Norm |
| Circuit Breakers | All In |
| AFCAU / RCU | Checked |
| Emergency L/G Handle | Down & Guarded |
| Parking Brake | On |
| Nose Wheel | As Required |
| Utility Hyd Accumulator Press | Green Arc |
| Cabin & Safety Items | Checked |
| ► Checklist complete. Standby Before Start |  |

| BEFORE START (R/D) |  |
| --- | --- |
| Gatilho: Preflight completo, preparação de cabine/cockpit finalizada e autorização obtida. |  |
| DC GPU (If utilizing, Batt On within 5 sec) | On |
| Battery (Min 25 Volts) | On |
| Triple Tachs | Flags Off |
| WCP | Test & Rearm |
| FADEC & GOV | No Lights |
| AC GPU (If utilizing, first Batt On) | On |
| AC PWR / DC PWR Lights On | Check |
| Emergency Bat | No Flow |
| Air Conditioning (If utilizing 115VAC) | As Required |
| Emerg / Heel Lights | Test & Arm |
| PAX Signs | On |
| Anticol / Position Lights | Red / On |
| AVCS | On |
| MGB / Engine Fire Test | Line & Fire Test |
| Chip Detectors / IGB.T / TGB.T | Test |
| VMS Lights (2 & 4 Lights) | Check |
| Sub Panel | Checked |
| Door Panel Lights | Check |
| Fuel Qtd & Transfer | Check / Off |
| Centre Console | On / Standby |
| HUMS Card | Checked |
| Perfo (MCDU) | Insert / Checked |
| Flight Controls | Full and Free, Centred |
| Collective & AP Hyd Cut-Out | Locked / Norm |
| ATC (Clearance) | Obtained |
| ► Checklist complete. Standby After Start |  |

| AFTER START (C/R) |  |
| --- | --- |
| Gatilho: Partida completa dos motores, parâmetros normais/estabilizados e GPU desconectada. |  |
| Cockpit / Cabin Air Cond | On / As Required |
| Engine Intake Anti-Ice | Tested & As Required |
| VMS & Systems | Checked Normal |
| Hydro Alt & Icedet | Tested & Off |
| AFCS Preflight Test | Performed / AP 1 & 2 On |
| Pitot & Windscreen Deice | On / As Required |
| Floats & Fuel Pumps | Tested & Off / Tested & On |
| Centre Console (FMS/TCAS/XPDR/SATCOM) | On / Standby |
| ISIS, Compass & Altimeters (QNH) | Set / Compare |
| WCP | No Lights |
| Mecanic / Chock | Clear / Removed |
| ► Checklist complete. Standby Before Taxi |  |

| BEFORE TAXI (C/R) |  |
| --- | --- |
| Gatilho: After Start completo, portas/passageiros prontos e intenção de táxi. |  |
| NR ILS | Off / NR 100% |
| Collective | Unlocked |
| Parking Brake | Released |
| Nose Wheel | Unlocked |
| Landing Light | A/R |
| Wheels Brakes | Check Pilot / Copilot |
| ADI / ISIS / HSI | Check |
| Fuel Quantity / Fuel Transfer | Check / Off |
| Take Off Briefing / VTOSS / V1 | Announced |
| ► Checklist complete. Standby Before Takeoff |  |

| BEFORE TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Linha de parada/pista. Offshore: Helideck e passageiros prontos. |  |
| Upper Modes | Preset |
| Anticolision / Flood Light | Both / On |
| NR | 100% |
| AP / WCP / VMS | On / No Lights / Normal |
| Heating | Off |
| Bleed OFS Pushbutton (If OAT ≥ +25°C) | Press |
| ACAS / Transponder | Auto / Alt |
| Nose Wheel (When aligned) | Locked |
| Parking Brake | As Required |
| Flight Instruments / Margin Power / VMS | Checked |
| ► Checklist complete. Standby After Takeoff |  |

| AFTER TAKEOFF (C/R) |  |
| --- | --- |
| Gatilho: Cruzando 500 ft AGL e livre de tráfego/obstáculos. |  |
| Landing Gear (300 ft / 80 kts) | Up / Lights Off |
| Bleed OFS Pushbutton | Depressed / Check VMS |
| WX Radar | On |
| Fuel (Qtd) / Transfer | Check Every 30 min / On |
| Landing Light | Off |
| Upper Modes | As Required |
| VMS | Normal |
| ► Checklist complete. Standby Cruise |  |

| CRUISE (C/R) |  |
| --- | --- |
| Gatilho: Nivelado na altitude de cruzeiro e estabilizado em rota. |  |
| WCP / VMS | No Lights / Normal |
| Radio / Nav | Set |
| Auto-Transfer | On |
| ► Checklist complete. Standby Descent |  |

| DESCENT (C/R) |  |
| --- | --- |
| Gatilho: A 5 NM do TOD (FMS) ou autorização de descida ATC. |  |
| Type & App Briefing | Perform |
| Alt A. / DH / QNH | Set / Adjust |
| Nav Aids Setting | Set |
| WCP / VMS | No Lights / Normal |
| Helideck Contact | Done |
| ► Checklist complete. Standby Landing |  |

| LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore/Offshore: A 5 NM do destino, na altitude de tráfego. |  |
| Fuel (Qtd) / Transfer | Check / Off |
| WX Radar | As Required |
| Bleed OFS Pushbutton (If OAT ≥ +25°C) | Press |
| Landing Gear | Down '3 Greens' |
| Landing Light | As Required |
| Parking Brake | As Required |
| Nose Wheel | Locked |
| WCP / VMS | No Lights / Normal |
| ► Checklist complete. Standby After Landing |  |

| AFTER LANDING (C/R) |  |
| --- | --- |
| Gatilho: Onshore: Pista livre. Offshore: Pousado, coletivo embaixo e equalizado. |  |
| Nose Wheel / Parking Brake | As Required |
| Landing Light | As Required |
| Anticolision | Red |
| Flood Light | Off |
| XPDR / TCAS / WX Radar | Std By |
| ► Checklist complete. Standby Shut Down |  |

| SHUT DOWN (C/R) |  |
| --- | --- |
| Gatilho: Aeronave estacionada, freio aplicado e calços colocados. |  |
| Parking Brake | On |
| Cyclic | Centered & Locked |
| Collective | Full Down & Locked |
| Collective & Cyclic Trim | Press |
| APM 1 / APM 2 | Off |
| MFDs | Off |
| Ldg Lgt / Sat Com / Floats | Off |
| Pitot / Windscreen / Bleed OFS | Off |
| WX Radar / Radios | Off |
| FMS / RADALT | Off |
| Booster Pumps | Off |
| Air Cond. / Heater / Eng Deice | Off |
| Eng Control Switches (Af. 1') | Idle 2 Sec. Then Stop |
| Arm Lever (Rotor Brake - Nr<45%) | Aft & Light On |
| Rotor Brake | Apply (Nr<45%) |
| A.Pump / Hyd Heating | Off |
| After Rotor Stops - PAX Signs | Off |
| Overhead Lighting Control Panel | All Switches Off |
| Fuel Qty | Check |
| N1/N2 Cycles (FADEC page) | Check |
| Main Hyd. Accumulator | Checked |
| Ldg Gear / Eng Safety Pins | Installed |
| M'ARMS Data Transfer (N1<5%) | Done |
| Flight Data | Acknowledge |
| Bat. Switch / Emerg Bat | Off / No Flow |
| ► Checklist complete. |  |